# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title : Supply of [……………………………]	p 1 /…

Ref. No.: ……………………………

Column 1-2 should be completed by the CARICOM Secretariat

Column 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the CARICOM  Secretariat shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item Number | 2. Specifications Required | Qtty | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- | --- |
| 1 | External Specifications System configuration: Containerized and waterproof (20ft.) Size: 20ft. x 8ft. x 8.6 ft. (Standard shipping container) Colour: White Construction: Stainless steel with anti-corrosive paint Doors: Stainless steel double or Single barn or swing doors Internal Specification Interior Capacity: ≥ 1000 cubic feet Insulation: Sandwich Panel, ≥ 100mm Interior Walls and Flooring: Stainless steel interiors and non-slip floors. (Finished with easy clean interiors and non-slip aluminium checker plate flooring) Storage capacity: ≥ 6MT Internal lighting included: Luminous output ≥ 300 Lux Fluorescent or LED Electrical Specifications Power Supply: 120/240 Vac, 60 Hz (1Ø, 3W) Cooling Unit: Inverter type Condenser: Copper Warranty 3 year Temperature Range: 0 to 150C with Temperature control system Compressor power: ≤ 2 kW Thermal Storage Capacity: 200MJ Refrigerant: 134a or 1234yf Power Supply Specification | 15 |  |  |  |
|  | Power Supply Specification Power Source: Onboard rapid deploying solar array & integrated Battery & Energy Management System (BEMS) Protection: Surge protection for DC/AC systems Solar photovoltaic modules: Tier 1 modules Solar module size: ≥ 340 Watt Monocrystalline Solar array capacity: ≥ 4,000 Watts peak Warrant on solar modules: 20-years Battery Capacity: ≥ 21 kWh Battery Technology: Maintenance-free OPzV or Lithium Iron Phosphate (LiFePO) Autonomy: Up to 16-hour battery feed before generator or grid power is needed Warranty on Battery: 7-years Energy Management System: Must allow the system to automatically switch to an alternative energy source (generator or grid power) when batteries reach pre-set discharge levels and automatically reverts when batteries are fully charged. Grid Operation: Grid-tie or islanded mode operation |  |  |  |  |